# Multi-Core MESI Cache Coherent Processor Simulator

Architecture Document

Computer Architecture Course (0512.4461) - Tel Aviv University

# 1. System Overview

This document describes the architecture of a multi-core processor simulator featuring four parallel cores with MESI cache coherence protocol. The system simulates a realistic pipelined processor environment with shared memory access.

## 1.1 Key System Components

- 4 Processor Cores: Each with a 5-stage pipeline, private instruction memory, and private data cache

- Instruction Memory (IMEM): Private SRAM per core, 1024 lines × 32 bits

- Data Cache: Private direct-mapped cache per core, 512 words, 8-word blocks

- Main Memory: Shared memory, 2²¹ words (21-bit address space)

- System Bus: MESI coherence protocol with round-robin arbitration

## 1.2 System Architecture Diagram

The following diagram illustrates the high-level system architecture:

┌─────────────┐  ┌─────────────┐  ┌─────────────┐  ┌─────────────┐

│   Core 0    │  │   Core 1    │  │   Core 2    │  │   Core 3    │

│  ┌───────┐  │  │  ┌───────┐  │  │  ┌───────┐  │  │  ┌───────┐  │

│  │ IMEM  │  │  │  │ IMEM  │  │  │  │ IMEM  │  │  │  │ IMEM  │  │

│  └───────┘  │  │  └───────┘  │  │  └───────┘  │  │  └───────┘  │

│  ┌───────┐  │  │  ┌───────┐  │  │  ┌───────┐  │  │  ┌───────┐  │

│  │ Cache │  │  │  │ Cache │  │  │  │ Cache │  │  │  │ Cache │  │

│  └───────┘  │  │  └───────┘  │  │  └───────┘  │  │  └───────┘  │

└──────┬──────┘  └──────┬──────┘  └──────┬──────┘  └──────┬──────┘

│                │                │                │

═══════╧════════════════╧════════════════╧════════════════╧═══════

SYSTEM BUS (MESI Protocol)

═══════════════════════════════════════╤═══════════════════════════

│

┌────────┴────────┐

│   Main Memory   │

│   (2²¹ words)   │

# 2. Register File Architecture

Each core contains 16 general-purpose registers, each 32 bits wide. The register file supports three simultaneous reads and one write per clock cycle.

## 2.1 Register Specifications

| Register | Name | Description |
| --- | --- | --- |
| R0 | $zero | Hardwired to zero. Writes are ignored. |
| R1 | $imm | Contains sign-extended immediate value from current instruction. Updated during decode. Read-only. |
| R2-R14 | $r2-$r14 | General-purpose registers for computation and data storage. |
| R15 | $ra | Return address register. Used by JAL instruction to store return address. |

## 2.2 Register File Timing

- Read Operations: Three ports allow simultaneous reads of rs, rt, and rd during Decode stage

- Write Operations: One write port, used during Write Back stage

- Timing Constraint: Data written in cycle N becomes readable in cycle N+1

- No Half-Cycle Operations: Unlike classic MIPS, this design uses full clock cycles for register access

# 3. Instruction Set Architecture

## 3.1 Instruction Format

All instructions use a uniform 32-bit encoding format:

| Bits 31:24 | Bits 23:20 | Bits 19:16 | Bits 15:12 | Bits 11:0 |
| --- | --- | --- | --- | --- |
| Opcode (8 bits) | rd (4 bits) | rs (4 bits) | rt (4 bits) | immediate (12 bits) |

## 3.2 Instruction Set Summary

### 3.2.1 Arithmetic and Logical Operations

| Opcode | Mnemonic | Operation |
| --- | --- | --- |
| 0 | ADD | R[rd] = R[rs] + R[rt] |
| 1 | SUB | R[rd] = R[rs] - R[rt] |
| 2 | AND | R[rd] = R[rs] & R[rt] |
| 3 | OR | R[rd] = R[rs] ¦ R[rt] |
| 4 | XOR | R[rd] = R[rs] ^ R[rt] |
| 5 | MUL | R[rd] = R[rs] × R[rt] |
| 6 | SLL | R[rd] = R[rs] << R[rt] (logical shift left) |
| 7 | SRA | R[rd] = R[rs] >> R[rt] (arithmetic shift right, sign extension) |
| 8 | SRL | R[rd] = R[rs] >> R[rt] (logical shift right) |
| 15 | JAL | R[15] = PC + 4; PC = R[rd][9:0] (Jump and Link) |

### 3.2.2 Branch Instructions

| Opcode | Mnemonic | Operation |
| --- | --- | --- |
| 9 | BEQ | if (R[rs] == R[rt]) PC = R[rd][9:0] |
| 10 | BNE | if (R[rs] != R[rt]) PC = R[rd][9:0] |
| 11 | BLT | if (R[rs] < R[rt]) PC = R[rd][9:0] |
| 12 | BGT | if (R[rs] > R[rt]) PC = R[rd][9:0] |
| 13 | BLE | if (R[rs] <= R[rt]) PC = R[rd][9:0] |
| 14 | BGE | if (R[rs] >= R[rt]) PC = R[rd][9:0] |

### 3.2.3 Memory and Control Instructions

| Opcode | Mnemonic | Operation |
| --- | --- | --- |
| 16 | LW | R[rd] = MEM[R[rs] + R[rt]] (Load Word) |
| 17 | SW | MEM[R[rs] + R[rt]] = R[rd] (Store Word) |
| 20 | HALT | Halt this core execution |

## 3.3 Branch Behavior and Delay Slot

This architecture implements a delay slot mechanism for branch instructions:

- The instruction immediately following a branch is always executed, regardless of whether the branch is taken

- Branch resolution occurs in the Decode stage

- If a branch instruction appears in the delay slot, it executes normally (compiler responsibility to avoid issues)

- Branch target is taken from the lower 10 bits of R[rd]

### 3.3.1 Branch with Data Hazard and Delay Slot

When a branch instruction has a data dependency:

- The branch stalls in DECODE stage waiting for the dependency to resolve

- While the branch stalls, FETCH also stalls (stall propagation rule)

- The delay slot instruction is not yet fetched while the branch is stalling

- Once the branch unstalls and moves to EX, the delay slot instruction is fetched

- The delay slot executes regardless of branch outcome

## 3.4 HALT Instruction Behavior

Important: HALT does NOT have a delay slot. Its behavior is different from branch instructions:

- HALT is detected in the DECODE stage

- The instruction currently in FETCH (PC+1) is cancelled/discarded - it will NOT execute

- HALT continues flowing through the pipeline: DECODE → EX → MEM → WB

- The pipeline drains with HALT being the last instruction

- After HALT completes WB, the core stops executing new instructions

- The core continues snooping the bus for cache coherence even after halting

Example pipeline trace showing HALT at PC=008:

Cycle 801: IF=009  ID=008(HALT)  EX=007  MEM=006  WB=005

Cycle 802: IF=---  ID=---        EX=008  MEM=007  WB=006  (009 cancelled)

Cycle 803: IF=---  ID=---        EX=---  MEM=008  WB=007

Cycle 804: IF=---  ID=---        EX=---  MEM=---  WB=008  (HALT completes)

# 4. Pipeline Architecture

Each core implements a classic 5-stage pipeline without bypass/forwarding logic. Data hazards are resolved through stalling.

## 4.1 Pipeline Stages

- FETCH (IF): Read instruction from IMEM at address PC. Instruction memory access completes in a single cycle.

- DECODE (ID): Decode instruction, read register operands (rs, rt, rd), sign-extend immediate to R1, resolve branches, detect HALT.

- EXECUTE (EX): Perform ALU operations, calculate memory addresses for load/store instructions.

- MEMORY (MEM): Access data cache for load/store operations. Cache hit completes in one cycle; miss triggers bus transaction.

- WRITE BACK (WB): Write results back to the register file.

## 4.2 Hazard Detection and Resolution

### 4.2.1 Data Hazards

Since there are no bypass paths, RAW (Read After Write) hazards are resolved by stalling:

- A dependent instruction is fetched and decoded normally

- During Decode, if a data dependency is detected, the instruction stalls

- The instruction remains stalled until the producing instruction completes Write Back

- Execution resumes in the cycle after Write Back completes (due to register file timing: write in cycle N, readable in cycle N+1)

### 4.2.2 Memory Hazards

Cache misses cause pipeline stalls:

- On cache miss in MEM stage, that instruction stalls

- All earlier pipeline stages (FETCH, DECODE, EXECUTE) also stall

- Write Back stage can continue to completion

- Stall continues until the cache miss is serviced

### 4.2.3 Stall Propagation Rule

General Rule: If stage Sn is stalled, all stages Sk where k < n are also stalled. Stages after Sn continue execution. This means stalls propagate backward through the pipeline.

## 4.3 Pipeline Timing Example

The following shows normal pipeline flow without hazards (first instruction fetched at cycle 1):

| Cycle | IF | ID | EX | MEM | WB |
| --- | --- | --- | --- | --- | --- |
| 1 | I1 | --- | --- | --- | --- |
| 2 | I2 | I1 | --- | --- | --- |
| 3 | I3 | I2 | I1 | --- | --- |
| 4 | I4 | I3 | I2 | I1 | --- |
| 5 | I5 | I4 | I3 | I2 | I1 |

# 5. Memory Hierarchy

## 5.1 Instruction Memory (IMEM)

| Parameter | Value |
| --- | --- |
| Type | Private SRAM per core |
| Width | 32 bits (one instruction) |
| Depth | 1024 lines |
| PC Width | 10 bits |
| Access Time | 1 clock cycle |

## 5.2 Data Cache Architecture

| Parameter | Value |
| --- | --- |
| Type | Private, direct-mapped |
| Total Size | 512 words |
| Block Size | 8 words |
| Number of Lines | 64 (512 / 8) |
| Hit Time | 1 clock cycle |
| Write Policy | Write-back, write-allocate |
| Coherence | MESI protocol |

## 5.3 Cache Address Breakdown

The 21-bit memory address is divided as follows:

| Field | Bits | Description |
| --- | --- | --- |
| Tag | 12 bits [20:9] | Identifies which memory block is cached |
| Index | 6 bits [8:3] | Selects one of 64 cache lines |
| Offset | 3 bits [2:0] | Selects one of 8 words within block |

## 5.4 Cache SRAM Structure

Each cache consists of two SRAM arrays:

### 5.4.1 DSRAM (Data SRAM)

- Width: 32 bits

- Depth: 512 words (64 lines × 8 words/line)

- Content: Cached data values

### 5.4.2 TSRAM (Tag SRAM)

- Depth: 64 lines

- Content per line: Tag (12 bits) + MESI state (2 bits)

TSRAM Entry Format:

| Bits [13:12] | Bits [11:0] |
| --- | --- |
| MESI State | Tag |

MESI Encoding: 0 = Invalid, 1 = Shared, 2 = Exclusive, 3 = Modified

Initial State: Both DSRAM and TSRAM are zeroed at simulation start (all lines Invalid).

## 5.5 Main Memory

| Parameter | Value |
| --- | --- |
| Address Space | 21 bits |
| Size | 2²¹ words (2,097,152 words) |
| Word Size | 32 bits |
| Access Granularity | Word-only (no byte addressing) |
| Read Latency | 16 cycles for first word, then 1 cycle per word |
| Write Latency | Instant (updates same cycle as Flush on bus) |

# 6. MESI Cache Coherence Protocol

The MESI protocol ensures cache coherence across all four cores. Each cache line can be in one of four states.

## 6.1 MESI States

| State | Code | Description |
| --- | --- | --- |
| Invalid | 0 | Line is not valid. Cache does not contain this data. |
| Shared | 1 | Line is valid and clean. Other caches may also have copies. Memory is up-to-date. |
| Exclusive | 2 | Line is valid and clean. No other cache has this data. Memory is up-to-date. |
| Modified | 3 | Line is valid and dirty. No other cache has this data. Memory is stale. |

## 6.2 Data Supply Rules

Important: Only a cache with the block in Modified state can supply data via Flush. Caches with Shared or Exclusive do NOT supply data.

- On BusRd/BusRdX: If any cache has block in Modified → that cache supplies data via Flush

- On BusRd/BusRdX: If no cache has Modified (only S/E/I) → Main memory supplies data (16-cycle latency)

- When Modified cache supplies data, main memory updates simultaneously from the Flush

## 6.3 State Transitions

### 6.3.1 Processor-Side Transitions (Local Core Actions)

- Read Miss (Invalid → Exclusive/Shared): Issue BusRd. Wait for Flush. If bus_shared=0 → Exclusive; if bus_shared=1 → Shared.

- Read Hit (Shared/Exclusive/Modified): No state change. Return data from cache in 1 cycle.

- Write Miss (Invalid → Modified): Issue BusRdX to get exclusive ownership. Wait for Flush. Write and go to Modified.

- Write Hit Exclusive (Exclusive → Modified): Silent upgrade (no bus transaction). Write data, transition to Modified.

- Write Hit Shared (Shared → Modified): Issue BusRdX (1 cycle). No Flush needed (data already in cache). Other caches invalidate. Transition to Modified.

- Write Hit Modified (Modified → Modified): No state change, no bus transaction. Write data to cache.

### 6.3.2 Bus-Side Transitions (Snooping Other Cores' Requests)

- Snoop BusRd while Shared: Set bus_shared=1. Remain in Shared. Do NOT supply data.

- Snoop BusRd while Exclusive: Set bus_shared=1. Transition to Shared. Do NOT supply data.

- Snoop BusRd while Modified: Flush data to bus (memory updates simultaneously). Set bus_shared=1. Transition to Shared.

- Snoop BusRdX while Shared: Transition to Invalid immediately.

- Snoop BusRdX while Exclusive: Transition to Invalid immediately.

- Snoop BusRdX while Modified: Flush data to bus (memory updates). Transition to Invalid.

## 6.4 Cache Block Eviction (Write-Back)

When a cache miss requires evicting a Modified block, two sequential bus transactions occur:

- Flush dirty block: Write the 8 words of the Modified block to main memory (8 cycles on bus)

- Request new block: Issue BusRd or BusRdX for the new block (1 cycle)

- Wait for response: 16 cycles latency if from memory

- Receive new block: 8 Flush cycles for the new block

Total eviction + fetch time: 8 + 1 + 16 + 8 = 33 cycles minimum (if new block comes from memory)

## 6.5 MESI State Transition Diagram

┌──────────────────┐

BusRd     │                  │     BusRdX

(snoop,M)    │    MODIFIED (3)  │    (snoop)

Flush→S     │                  │    Flush→I

│        └────────▲─────────┘        │

│                 │ Write Hit          │

▼                 │ (silent E→M)       ▼

┌──────────────────┐        │        ┌──────────────────┐

│                  │  Write │        │                  │

│   SHARED (1)     │────────┼───────>│  EXCLUSIVE (2)   │

│                  │ BusRdX │        │                  │

└────────┬─────────┘(no Flush)       └────────┬─────────┘

▲    │                                    │    ▲

│    │ BusRdX                      BusRdX │    │

BusRd   │ (snoop)                    (snoop) │  BusRd

(shared=1)│                                    │(shared=0)

│    ▼                                    ▼    │

│  ┌──────────────────────────────────────────┐│

└──│                INVALID (0)               │┘

└──────────────────────────────────────────┘

# 7. System Bus Architecture

## 7.1 Bus Signals

| Signal | Width | Description |
| --- | --- | --- |
| bus_origid | 3 bits | Transaction originator: 0-3 = Core 0-3, 4 = Main Memory |
| bus_cmd | 2 bits | Command: 0 = No command, 1 = BusRd, 2 = BusRdX, 3 = Flush |
| bus_addr | 21 bits | Word address in main memory |
| bus_data | 32 bits | Data word being transferred |
| bus_shared | 1 bit | Set to 1 if any cache has the data (during BusRd snooping) |

## 7.2 Bus Commands

| Command | Code | Description |
| --- | --- | --- |
| No Command | 0 | Bus is idle (but may be locked waiting for Flush response) |
| BusRd | 1 | Read request. Core wants to read a block for loading. |
| BusRdX | 2 | Read-exclusive request. Core wants exclusive ownership for writing. |
| Flush | 3 | Data response. Sends one word per cycle. Used by memory or Modified cache. |

## 7.3 Bus Arbitration

The bus uses fair Round-Robin arbitration when multiple cores request access simultaneously:

- The core that most recently received bus access has the lowest priority

- Priority rotates among cores 0 → 1 → 2 → 3 → 0 ...

- A core cannot get bus access while the bus is busy or a previous BusRd/BusRdX awaits its Flush response

## 7.4 Bus Transaction Timing

### 7.4.1 Read Transaction (BusRd/BusRdX with Flush Response)

When data must be fetched (from memory or Modified cache):

- Cycle 0: BusRd or BusRdX appears on bus with address

- Cycles 1-15: Bus idle (bus_cmd=0) but LOCKED - no new transactions allowed

- Cycle 16: First Flush word arrives (if from memory); Modified cache may respond earlier

- Cycles 16-23: 8 Flush words, one per cycle, starting from block-aligned address

- Total: For N words from memory: N + 15 cycles (16 latency + N-1 additional words)

### 7.4.2 Upgrade Transaction (BusRdX from Shared, No Flush)

When a core has data in Shared and wants to write:

- Cycle 0: BusRdX appears on bus

- Other caches snoop and invalidate their copies immediately

- No Flush needed: Data is already in the requesting cache

- Total: 1 cycle on bus

### 7.4.3 Write-Back Transaction (Flush Only)

When evicting a Modified block or responding to BusRd/BusRdX:

- 8 consecutive Flush cycles, one word per cycle

- Each Flush includes the word address in bus_addr

- Main memory updates simultaneously (instant write)

- Total: 8 cycles

### 7.4.4 Bus Trace Example

Example from bustrace.txt - Core 2 reads address 0x0F:

Cycle  origid  cmd  addr    data      shared

5      2       1    00000F  00000000  0       <- BusRd from Core 2

6-20   (bus_cmd = 0, bus locked waiting for response)

21     4       3    000008  00000000  0       <- Flush word 0 from memory

22     4       3    000009  00000000  0       <- Flush word 1

...    ...     ...  ...     ...       ...

28     4       3    00000F  00000064  0       <- Flush word 7 (requested addr)

# 8. Simulator Implementation

## 8.1 Execution Model

- Initialization: All cores start at PC = 0. All pipeline stages are empty (---). All DSRAM and TSRAM are zeroed.

- First Fetch: Occurs at cycle 1.

- Parallel Execution: All four cores run simultaneously and independently.

- Halt Behavior: When HALT reaches DECODE, the instruction in FETCH is cancelled. HALT flows through to WB, then core stops. Core continues snooping after halt.

- Termination: Simulator exits when all cores have halted and all pipelines are empty.

## 8.2 Memory Output Behavior

Important: The simulator does NOT automatically flush Modified cache blocks at the end of simulation.

- memout.txt reflects the actual state of main memory

- If a block is Modified in cache but never written back, it will NOT appear in memout.txt

- The assembly program is responsible for forcing write-backs (e.g., via conflict misses)

- This will be tested - do not auto-flush at simulation end!

## 8.3 Command Line Interface

The simulator accepts 27 file path arguments:

sim.exe imem0.txt imem1.txt imem2.txt imem3.txt memin.txt memout.txt regout0.txt regout1.txt regout2.txt regout3.txt core0trace.txt core1trace.txt core2trace.txt core3trace.txt bustrace.txt dsram0.txt dsram1.txt dsram2.txt dsram3.txt tsram0.txt tsram1.txt tsram2.txt tsram3.txt stats0.txt stats1.txt stats2.txt stats3.txt

If no arguments are provided, default filenames are used from the executable's directory.

## 8.4 Input Files

| File | Description |
| --- | --- |
| imem0-3.txt | Instruction memory for each core. Hex format, 8 digits per line, starting at address 0. |
| memin.txt | Initial main memory contents. Hex format, 8 digits per line, starting at address 0. |

## 8.5 Output Files

| File | Description |
| --- | --- |
| memout.txt | Final main memory contents (NOT including unflushed Modified blocks). |
| regout0-3.txt | Final register values (R2-R15) for each core. |
| core0-3trace.txt | Pipeline trace: cycle, PC in each stage, register values. |
| bustrace.txt | Bus transactions (only when bus_cmd ≠ 0). |
| dsram0-3.txt | Final DSRAM contents for each cache. |
| tsram0-3.txt | Final TSRAM contents for each cache. |
| stats0-3.txt | Statistics for each core. |

## 8.6 Statistics Counters

| Counter | Description |
| --- | --- |
| cycles | Number of clock cycles the core was running until halt. |
| instructions | Number of instructions that completed (reached Write Back stage). |
| read_hit | Number of cache read hits. |
| write_hit | Number of cache write hits. |
| read_miss | Number of cache read misses. |
| write_miss | Number of cache write misses. |
| decode_stall | Total number of cycles stalled in decode stage (data hazards). |
| mem_stall | Total number of cycles stalled in memory stage (cache misses). |

# 9. Test Programs

Three test programs are required to validate the simulator:

## 9.1 Counter Test (counter/)

- Description: All 4 cores increment a shared counter at memory address 0

- Pattern: Cores take turns in order: Core 0 → Core 1 → Core 2 → Core 3 → Core 0 ...

- Iterations: Each core increments 128 times (total = 512 increments)

- Validation: Final value at address 0 should be 512

- Write-back: Must force write-back to main memory via conflict miss (required for memout.txt)

## 9.2 Serial Matrix Multiplication (mulserial/)

- Description: 16×16 matrix multiplication on Core 0 only

- Matrix A: Addresses 0x000 - 0x0FF (256 elements)

- Matrix B: Addresses 0x100 - 0x1FF (256 elements)

- Result C: Addresses 0x200 - 0x2FF (256 elements)

- Layout: Row-major order (C-style): first row left-to-right, then next row, etc.

- Algorithm: C[i][j] = Σ(A[i][k] × B[k][j]) for k = 0 to 15

## 9.3 Parallel Matrix Multiplication (mulparallel/)

- Description: Same 16×16 matrix multiplication distributed across all 4 cores

- Goal: Minimize total execution time through parallel computation

- Strategy: Each core computes a portion of the result matrix (e.g., 4 rows each)

- Scoring: Lower total cycle count receives higher grade

# 10. Key Implementation Considerations

## 10.1 Critical Timing Points

- Register writes take effect next cycle (write in N → readable in N+1)

- Branch resolution in Decode with delay slot always executed

- Stalled instructions resume the cycle AFTER dependency resolves

- HALT has NO delay slot - next instruction is cancelled

- Bus transactions block all cores' cache accesses until complete

- Memory reads: 16-cycle latency; Memory writes (Flush): instant

## 10.2 Common Pitfalls

- Forgetting that R0 is always 0 and R1 is the sign-extended immediate

- Auto-flushing Modified blocks at simulation end (DO NOT do this)

- Incorrect MESI transitions: only Modified supplies data, S/E do not

- Not properly implementing round-robin arbitration

- Miscounting stall cycles vs. stall events (should count cycles)

- Forgetting that halted cores must still snoop the bus

- BusRdX from Shared: requires only 1 cycle (no Flush), not 16+

- Counting cancelled instruction after HALT as executed

- Not fetching delay slot when branch stalls on data hazard